### OBJECTIVE:

- To interface an ADC to the 8051.

### REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller and Embedded Systems,” Chapter 13.

- Look for the MAX1112 data sheet in The Maxim Corp. web site:

www.maxim-ic.com

MATERIALS:

- 8051 trainer

- 8051 assembler

- MAX1112 ADC

### ACTIVITY

Connect the MAX1112 to the 8051 as shown in chapter 13 in the textbook. Then assemble and run a program to display the data on the LCD or send it to a PC screen through the serial port of the 8051. Notice that the ADC output is between 0 to FFH, where FFH is for a full-scale input. However, this must be converted to decimal and then to ASCII in order to be displayed on the LCD. As you change the potentiometers, the output should change, indicating the value of the analog input.

The next lab will build upon the connections you have made in this lab.

Name:						Date: 				Class:

1. Indicate the direction of pins SCLK, DIN, and CS from the point of view of the 8051.

2.  Give the steps for selecting channel, converting data, and getting the data out of the MAX1112. State the status of the CS and SCLK pins in each step.

3.  Assume that REFIN is connected to 1.28 V. Find the following.

(a) step size

(b) maximum range for Vin

(c) D7 - D0 values if Vin = 1.2 V

(d) Vin if D7 - D0 = 11111111

(e) Vin if D7 - D0 = 10011100

4.  Assume that REFIN is connected to 1.9 V. Find the following.

(a) step size

(b) maximum range for Vin

(c) D7 - D0 values if Vin =  2.7 V

(d) Vin if D7 - D0 = 11111111

(e) Vin  if D7 - D0 = 11011101

5. The MAX1112 is a(n) ____-bit converter with  ____analog input channels.

6. To get step size of 2 mV, what is the value for Vref?