### OBJECTIVES:

- To program the hardware interrupt of the 8051.

### REFERENCE:

- Mazidi and Mckinlay, “The 8051 Microcontroller …,” Chapter 11.

### MATERIALS:

- MDE8051 trainer

- 8051 Keil Compiler

- Square wave generator

### ACTIVITY 1

Write an 8051 program to get data from a single bit of P1.0 (connected to DIP SW) and send it to P2.7 (connected to LED) continuously while an interrupt can do the following: Connect the INT0 (or INT1) pin to a normally high debounce SW and any time the switch is pressed, a single LED is turned on.

### ACTIVITY 2

Write an 8051 program to get data from a single bit of P1.0 (connected to DIP SW) and send it to P2.7 (connected to LED) continuously while an interrupt will do the following: A square wave is connected to the INT0 (pin 3.2) and any time a H-to-L pulse comes in a single LED is turned on. The rate of "On" and "Off" is the rate of the square wave. In this activity you can also use INT1.

Name:						Date: 				Class:

1.  Name all of the hardware interrupts in the 8051 and their vector table addresses.

2. On reset, INT0 (and INT1) are _________ (edge, level) triggered.

3. On reset, which interrupt has the highest priority?

4. True or False. There is only a single interrupt for the serial data transfer.