### OBJECTIVES:

- To review memory concepts.

- To examine the memory map of the Dragon12+ Trainer board

### REFERENCE:

- Mazidi & Causey “HCS12 Microcontroller and Embedded Systems,” Chapters 0 1, and 2.

### MATERIALS:

- Dragon12 Plus Trainer manual

### ACTIVITY 1A

Dragon12 plus Trainer board package comes with a CDROM containing many ref. information including user’s manual. Locate the Dragon12-Plus user’s manual on the CD ROM or find the latest one on the www.evbplus.com website.

Now, examining the manual we will see that the Dragon12+ board uses the MC9S12Dx256 microcontroller chip from Freescale, where x can be P,T,J, or G version. The manual also shows the memory map of the MC9S12Dx256 chip. You can also locate the memory map of this microcontroller on the www.freescale.com.  Examining the memory map of MC9S12Dx256 under NORMAL SINGLE CHIP we see the following:

Reg./Memory			Address range used ($ mean in hex)

REGISTERS  			$0000 - $03FF

EEPROM			$0400 - $0FFF

RAM				$1000 – $3FFF

Flash ROM			$4000 - $FFFF

The above addresses assignments are the defaults upon RESET and can be mapped into different locations. We use the above default addresses throughout the Labs. Now, calculate and fill in the total memory space (in bytes and K bytes) assigned for each section of the above addresses.

Reg./Memory			Address range		Bytes		K bytes

REGISTERS  			$0000 - $03FF		…….		……

EEPROM			$0400 - $0FFF		…….		…….

RAM				$1000 – $3FFF	…….		……..

Flash ROM			$4000 - $FFFF	…….		……..

Name:						Date: 				Class:

1. Give two differences between EEPROM and Flash memory?

2. Give three differences between SRAM  and Flash memory?

3. For Questions 1 and 2, indicate the use of each memory in microcontroller. In other words, state which one is used for Code, Data, and scratch memory.

4. State the number of address and data pins for 64Kx8 Flash memory.

5. State the number of address and data pins for 4Kx8 SRAM  memory

6. State the number of bytes and K bytes of memory space for the address range of 4000H – 7FFFH (H for Hex)

7. How much on-chip Flash memory do we have in the MC9S12Dx256? Give its starting address upon Reset.

8. State the number of K bytes for address space of 4000H – FFFFH.

9. How much on-chip SRAM memory do we have in the MC9S12Dx256? Give its starting address upon Reset.

10. How much on-chip EEPROM do we have in the MC9S12Dx256?  Give its starting address upon Reset.