### OBJECTIVES:

- To interface a stepper motor to PIC.

- To write a program to control the angle and direction of stepper motor rotation by the user.

# REFERENCES:

- Mazidi, McKinlay, and Causey, “PIC Microcontroller and Embedded Systems,” Chapter 17.

### MATERIALS:

- PIC Trainer

- MPLAB

- stepper motor module

### ACTIVITY 1

Connect the stepper motor module to PIC Trainer, or build the circuit shown in Chapter 17 of the textbook. Then write and run a program to rotate it clockwise continuously. If you are setting up the circuit, use the following steps to figure out the stepper motor leads.

1. Use an ohmmeter to measure the resistance of leads. This should identify the COM leads

A through D winding leads.

2. The common wires are connected to the positive side of the motor's power supply. In

many motors, +5V is sufficient.

3. The four leads of the stator winding are controlled by four bits of the PIC port. However, since     the PIC lacks sufficient current to drive the stepper motor windings, we must use a driver such     as the ULN2003(ULN2008) to energize the stator. Instead of the ULN2003 (ULN2008), we         could have used transistors as drivers, as shown in Chapter 17. However,

notice that if transistors are used as drivers, we must also use diodes to take care of

inductive current generated when the coil is turned off. One reason that the ULN2008 is

preferable to the use of transistors as drivers is the fact that the ULN2008 has an internal

diode to take care of back EMF. Change the delay to see the speed of rotation.

### ACTIVITY 2

After making sure that Activity 1 works, write and run a program to rotate clockwise 180 degrees, stop for a second, then go counter clockwise to its original position.

### ACTIVITY 3

After making sure that Activity 1 works, write and run the following:

1. Connect a buffered switch to RB1 and use it for clockwise or counter clockwise. Use

RB1=0 for CW and RB1=1 for CCW.

2. After the user has made a choice, the motor will rotate accordingly.

You can also use RB2 and RB3.pins to choose the degree of rotation.

Name:						Date: 				Class:

1. What is a step angle?  Define steps per revolution.

2. If a given stepper motor has a step angle of 5 degrees, find the number of steps per

revolution.

3.  Give the four sequences for counter clockwise if it starts with 10011001 (binary).

4. Using the "RLNCF" instruction, show the four-step sequences if the initial step is 0011

(binary).

5. Give the number of times the four-step sequence must be applied to a stepper motor to

make a 100-degree move if the motor has a 5-degree step angle. Also fill in the

characteristics for your motor below.

Step angle  ______________	Degree of movement per 4-step sequence  ________

Steps per revolution     _____          Number of rotor teeth  ______________