LAB 1-17

STRING OPERATIONS

OBJECTIVE

To use the STOS and LODS instructions in a data transfer program

REFERENCES

Mazidi, Volumes I & II: Chapter 6, Section 6.2

MATERIALS

80x86 IBM (or compatible) computer

MASM (or compatible) assembler

ACTIVITY 1

Write a program that clears the screen.

Add instructions to display a prompt such as "Type in the message" and wait for the user to respond.

Add instruction to get the user's typed-in string and save it.

Add instructions to display another prompt such as "Would you like to convert from the upper case to lower case Y/N?".

If the user types in "Y" or "y" then the message typed in by the user is converted to lower case and displayed; otherwise, the program ends and returns to DOS. The conversion must be done using the LODS and STOS instructions.  This can done by modifying Program 3-4 in Chapter 3 of the textbook.

WORKSHEET QUESTIONS

1. For  the instruction STOS, state the implied source and destination operands. For the instruction LODS, state the implied source and destination operands.  Did you use a REP prefix in your program? What condition(s) will cause the repeat to stop?

2. Attach a screen print (use Shift-PrtScr) of the screen after the user's input has been converted to upper case and displayed.