LAB 2-5

BOX DRAW PROGRAM

Write a program to draw three concentric boxes.  Draw the outer box first, from the top left corner, clockwise, then draw the middle box counterclockwise, and finally the inner box clockwise.  Insert a SLOW module (shown below) to make the system draw the boxes in slow motion.  Choose 3 different colors for your boxes.  The following shows a structure that could be used to define  the starting and ending coordinates of a line. It is followed by four definitions of lines with their initial data. These four lines will comprise a box when drawn. Refer to Appendix C in the textbook for more information about structures.

LINE		STRUC		;begin line structure

ROW_STRT	DW	?

COL_STRT	DW	?

ROW_END	DW	?

COL_END	DW	?

LINE		ENDS	;	end line structure

LINE1LINE <10,10,10,309>	;starts at 10,10 ends at 10,309

LINE2LINE <10,309,189,309>	;starts at 10,309 ends at 189,309

LINE3LINE <189,10,189,309>

LINE4LINE <10,10,189,10>

Write 4 macros: one to draw a horizontal line from left to right, one to draw a horizontal line from right to left, one to draw a vertical line from top to bottom, one to draw a vertical line from bottom to top.  The following is a macro that would use the data definitions above to draw a horizontal line from left to right.

HORIZ_L_R	MACRO LINE_H

LOCAL  DRAW_L_R

;this macro draws a horizontal line from left to right

MOV BX,LINE_H.COL_END

MOV  AH,0CH		;draw pixel option

MOV  AL,10B		;pixel value

MOV  DX,LINE_H.ROW_STRT;DX = row

MOV  CX,LINE_H.COL_STRT;CX = column

DRAW_L_R:INT  10H	;draw a pixel

SLOW			;slow down

INC  CX		;move 1 column to the right

CMP  CX,BX		;is it the last column?

JB  DRAW_L_R	;if not, continue drawing

ENDM

The above macro would be called with LINE1 as follows:

HORIZ_L_R LINE1

There are many ways to write the SLOW macro; here is one way:

SLOW	 MACRO

LOCAL SLOW_LOOP

;this macro inserts a delay into program by counting down from FFFF to 0

;this is slow enough for a 50 MHz CPU, it may be too slow for other CPUs

MOV SI,0FFFFH

SLOW_LOOP:SUB SI,1

JNZ SLOW_LOOP

ENDM